ALGORITHM:

Stopping Sight Distance (SSD)

Step 1: Start.

Step 2: Read input: Ask the user to enter                     Speed V in km/h.

Step 3: Read input: Ask user to enter reaction time t in seconds (typical 2.5 s if unknown).

Step 4: Read input: Ask user to enter coefficient of friction f (dimensionless).

Step 5:Read input: Ask user to enter grade G as a decimal (e.g., +0.02 for +2% uphill, −0.03 for −3% downhill).

Step 6: Validate inputs: Ensure V ≥ 0, t > 0, f > 0. If invalid show error and stop.

Step 7: Compute lag (thinking) distance LD:

LD = 0.278 × V × t.

Step 8: Compute braking denominator:

denom = 254 × (f + G).

If denom ≤ 0, show error (unsafe/invalid friction+grade) and stop.

Step 9: Compute braking distance BD:

BD = (V × V) / denom.

Step 10: Compute SSD:

SSD = LD + BD.

Step 11: Output: Display LD, BD, SSD with units (meters).

Step 12: End.

Soil Bearing Capacity Checker —

Step 1: Start.

Step 2: Read input: Ask user to enter applied total vertical load P in kN.

Step 3: Read input: Ask the user to enter footing area A in m².

Step 4: Validate inputs: Ensure P ≥ 0 and A > 0. If invalid show error and stop.

Step 5: Compute ultimate bearing pressure q = P / A (kN/m²).

Step 6: Apply factor of safety (optional): If user supplies FS (typical 2.5 -- 3), compute allowable / safe bearing capacity SBC = q / FS. If no FS, assume SBC = q and note it is unfactored.

Step 7: Classify soil (example thresholds):

If SBC ≤ 100 kN/m² → “Weak soil”

If 100 < SBC ≤ 200 kN/m² → “Medium strength”

If SBC > 200 kN/m² → “Strong soil”

(Mention these are example thresholds — users may change them.)

Step 8: Output: Display q, SBC (if FS used), and classification. Include units (kN/m²).

Step 9: End.

Water Tank Volume Calculator —  (Menu + two shapes)

Step 1: Start.

Step 2: Display menu:

1. Rectangular tank

2. Cylindrical tank

Step 3: Read input: User selects choice (1 or 2).

Step 4:Validate choice: If invalid show error and return to menu or stop.

If choice = 1 (Rectangular):

5. Read length L (m).

6. Read width W (m).

7. Read height H (m).

8. Validate L>0, W>0, H>0 — if not, show error and stop.

9. Compute Volume V = L × W × H (m³).

10. Optionally compute litres = V × 1000.

11. Display V (m³) and litres.

12. End.

If choice = 2 (Cylindrical):

5. Read radius r (m) or diameter d (m). If diameter given, set r = d/2.

6. Read height h (m).

7. Validate r>0 and h>0 — if not, show error and stop.

8. Compute Volume V = π × r² × h (use π = 3.14159265).

9. Optionally compute litres = V × 1000.

10. Display V (m³) and litres.

11. End.